EHS

Environmental Health and Safety

Handling IPA (Isopropyl Alcohol)

Standard Operating Procedure

Name:

Date:

Revision Number:

Date of Revision:

*Attach description of process that involves the use of IPA.*

1.0  Hazards Associated with IPA:

IPA is a flammable liquid as well as an eye and respiratory tract irritant.  Exposure may cause drowsiness or dizziness.  IPA has an ACGIH Threshold Limit Value (TLV) of 200 ppm.

2.0  Controls:

2.1  Engineering Control:

- Utilize a fume hood.

2.2  Administrative Controls:

- Do not work alone in the lab.  Implement the buddy system.

- Take the online EHS lab safety training annually located at this link - https://www.uml.edu/eem/ehs/lab-safety/.

  - Follow instructions for registering your training after you receive your quiz grade which is step 4 in the instructions.  Without registering your training, you will not receive an email with your EHS lab safety training certificate attached.  You will also not appear in the EHS records for taking this online training so it is very important to follow step 4.

- Know the location of the emergency shower and eyewash station.

- Review this SOP and applicable safety data sheets as part of your laboratory-specific training on handling toluene.

- Keep a hard copy of the safety data sheet (SDS) for IPA in section 6 of the CHP Notebook.

- Place the IPA SOP in section 7 of the CHP Notebook.

- Wash hands thoroughly with soap and water after removing gloves.

2.3  Personal Protective Equipment:

- Safety goggles are required if there is a splash hazard.  Safety glasses are required at a minimum.

- Face shield required if there is a splash hazard to the face.

- Laboratory coat

- Disposable nitrile gloves

- Closed toe shoes and pants

3.0  Precautions:

- Avoid contact with eyes and skin.

- Avoid inhalation of vapor or mist.

4.0  Storage:

- Store in a well-ventilated location.

5.0  Disposal:

Place the UMass Lowell Hazardous Waste Label on the container and fill out the label.  Full and/or dated containers of hazardous waste are picked up by EHS during the weekly inspection checks for satellite accumulation areas or upon request by submitting a pick-up request online at https://www.uml.edu/eem/ehs/hazardous-materials/hazardous-waste-management/pickups.aspx.

NOTE:  Gloves, KimWipes and other solid waste that has been in contact with IPA but the solvent has been consumed may be disposed of in the trash.

6.0  Emergency Procedures:

6.1  Spills

For spills that occur inside of a fume hood (< 100 milliliters)

- If lab group has received proper training and a proper spill kit is available, the spill may be cleaned up by the lab group if the spill occurs inside of a fume hood.

- If the lab group has not received proper training and a spill kit is not available, please evacuate the lab and call 44911.

For spills that occur outside of a fume hood

- Without training on how to clean up a spill, evacuate the lab, post a restriction on the lab door and call UMass Lowell Police at extension 44911.

- Stay close by to answer questions when EEM-EHS and emergency response personnel arrive.

- With training on how to clean up a spill, and the quantity of liquid is small (less than 1 liter); you may clean up the spill with absorbent pads or paper towels.  Place contaminated spill material in a container that can be sealed and then call 42543 for assistance.

6.2  First Aid

For eyes

- Irrigate the eyes for 15 minutes, holding eyelids apart.

- Call extension 44911 or 978-934-4911 from a cell phone to seek medical assistance and then can keep track of the length of time eyes are being irrigated.

For skin

- For full body exposure, remove contaminated clothing and go under the emergency shower for 15 minutes.  If bare hands and/or arms are contaminated, rinse area with soap and water for 15 minutes.

- Call extension 44911 or 978-934-4911 from your cell phone immediately to seek medical assistance.

For inhalation

- Remove to fresh air.  Seek medical attention immediately.  Call extension 44911 or 978-934-4911 from a cell phone.

6.3 Fire:

- Evacuate the lab, pull the nearest fire alarm pull station and then go to a safe area and call extension 44911 or 978-934-4911 from a cell phone.  Follow the fire safety evacuation plan.

NOTE:  All work-related injuries must be reported immediately to Human Resources (HR) by calling extension 43560.  An Accident/Near Miss Incident Form must be filled out.  (This form is available on-line at website.)

*The buddy, supervisor, or Principal Investigator may fill out the Incident/Injury Report Form while the injured employee follows first aid procedures and seeks medical attention.